Pieces: ..............

Stainless steel floor tank

Dimensions 400x400x150 mm high, with 4 adjustable feet + 40 mm.

Inset removable stainless steel lid 15 mm,

with waterproof removable inner lid,

Tightness of the closed lid IP68,

Lid lockable inside with padlock,

Suitable for outdoor use, wheel load according to DIN 1072/SLW60 = 15 to.

1 socket compartment containing

Assembly modules: 	Lead-in modules behind:

1 M3- 1 M3-                   1 M3- 1 M3-

1 M3- 1 M3-                   1 M3- 1 M3-

1 M3- 1 M3-                   1 M3- 1 M3-

1 M3- 1 M3-                   1 M3- 1 M3-

1 Separate compartment containing

1 M3- lateral (water, supply line, or cover)

1 M3- lateral (water, supply line, or cover)   > select one

1 M3- bottom (drain, or cover)

1 M8-004 bottom (distributor 18 TE instead of water)   > select one

Place floor tank in concrete

according to the assembly instructions supplied,

Align level with FFB, if necessary in tile pattern.

Equip distributor with FI and LSS according to capacity.

Wire sockets and distributors in accordance with regulations,

Check direction of rotation and protective measures.

Connect tap water loop and check for leakage.

Connect wastewater to sewer via floor tank siphon (HT pipe DN50).

Drain waste water via soil tank siphon to the side in a gravel bed for infiltration.

>   select one

Fill the pit with concrete up to the lower edge of the cover seat for load transfer.

Dispose of superfluous excavated soil, leave the installation site swept clean.

Brand: StarConnex

Type: UMKIRCH or equivalent.